### Total: ____/40

Report By: Yao Xinchen

Lab Partner: Rebecca Rauschmayer, Liu Shiyu

Lab TA:  Junjie Gao

Section:  AB2

## Question 1.  ___/15

### Theoretical/Experimental Results						___/5

Mp = (MaxValue – SteadyState)/SteadyState * 100

|  | Mp Theory % | Mp Expmt % | tr Theory (s) | tr Expmt (s) | ts Theory (s) | ts Expmt (s) |
| --- | --- | --- | --- | --- | --- | --- |
| 2.0 | 0.00 | 0.00 | 8.20 | 8.02 | 11.60 | 11.14 |
| 1.5 | 0.00 | 0.00 | 5.85 | 5.80 | 8.30 | 8.32 |
| 1.0 | 0.00 | 0.10 | 3.35 | 3.32 | 5.00 | 4.76 |
| 0.8 | 1.52 | 1.77 | 2.50 | 2.54 | 3.68 | 3.38 |
| 0.7 | 4.60 | 5.62 | 2.16 | 2.28 | 3.02 | 4.86 |
| 0.5 | 16.30 | 17.24 | 1.62 | 2.04 | 6.28 | 5.32 |
| 0.3 | 37.23 | 40.37 | 1.30 | 1.88 | 10.14 | 10.62 |
| 0.2 | 52.66 | 70.56 | 1.21 | 1.86 | 15.08 | 17.46 |

Table 1: Theoretical/Experimental Results

Attach one sample plot from your StepResponseMetrics file that shows how you obtained the experimental results for one of the values of .

### Comparison of Theoretical vs. Experimental Results			___/5

Hint:  Does it look like the theoretical equations on page 11 of the lab manual match the experimental values?

Put Discussion Here

Yes, they match.

### Discussion of variation with  of Mp, ts, tr					___/5

Put Discussion Here

When  decreases, Mp increases, tr decreases, and ts first decreases and then increases.

## Question 2.  ___/15

### Effect of  on Pole Locations	(Derive Equation and Explain)	___/5

Put Discussion of ’s Effect Here. Include the equation of the two pole locations in terms of  (you may assume ωn = 1). Include either a sketch/graph of the pole locations as  increases, or a description of what this graph would look like.

s1, 2= -ζ±j1-ζ2

As  increases, the poles go more left on the left half of the unit circle, and then go left on the x axis.

### Effect of Pole Locations on Mp, ts, tr for an Underdamped System												___/5

Hint: An underdamped system has   ____

As  increases, the poles do____ which makes Mp, ts, tr do _______

(Double Hint: moving the poles causes two different effects on ts)

An underdamped system has  less than 1.

As  increases, the poles go left on the left half of unit circle, which makes Mp smaller, ts smaller and tr greater

### Effect of Pole Locations on Mp, ts, tr for an Overdamped/Critically Damped System									___/5

Hint: An over-damped system has   ____

A critically damped system has   ____

As  increases, the poles do____ which makes Mp, ts, tr do _______

An overdamped system has  greater than 1.

As  increases, the poles go left on the x axis, which makes Mp be zero, ts greater and tr greater

## Question 3.  ___/10

Investigate the effects of approximating an overdamped 2nd order system with a 1st order system. The approximation will be done by using a transfer function with only the pole that is closer to the origin, pmin.

The response speed would be underestimated. The settling time would be overestimated. The rise time would be underestimated. And the overshoot would be overestimated.

### Similarities/Differences on Overdamped 2nd-Order system to a 1st-Order System with the less negative of the 2nd-Order’s poles	___/6

Plot the step responses for the 2nd order systems and their 1st order approximations for    = 1.5,  = 5, and  = 40. Assume ωn = 1. How are the step responses of the 1st order approximations similar to and different from the step responses of the original 2nd order systems?

The 1st approximations’ step responses have a similar shape as the original ones’. But they rise faster.

### Effect of magnitude of  on the accuracy of the approximations	___/4

How does   affect the accuracy of the 1st order approximations?

The greater  is, the higher the accuracy is.

## Attachments (3)

- Plots obtained during lab

- Sample response with relevant points for calculating Mp, ts and tr marked

- Step Responses comparing 2nd order systems and 1st order approximations

Plots obtained during lab:

Sample response with relevant points for calculating Mp, ts and tr marked:

Step Responses comparing 2nd order systems and 1st order